# Domestic Aviation — Phase 1 Calibration Report

Family: domestic_aviation   |   Generated: 2026-05-02   |   Unit: million_pkm

# 1. Calibration Basis

Demand anchor. The 2025 baseline demand is anchored at 75,500 million_pkm.

Energy calibration. Total final energy at the 2025 anchor is 85.0 PJ (AES 2023-24). This gives an energy intensity of 1,126.0 GJ/million_pkm. Input fuels: aviation_turbine_fuel.

Emissions calibration. Scope 1 CO2e at the anchor equals 6.1 MtCO2e (Cat 1A3a domestic aviation). This uses NGA 2024 emission factors and IPCC AR5 GWP100. Electricity-related emissions are excluded (scope 2 boundary, per A003).

# 2. Technology States

## Conventional jet

State ID: domestic_aviation__conventional_jet. Classification: incumbent. Max share range: 100% (2025) to 60% (2050). Cost: AUD 0.05/million_pkm (2024 real). Energy trajectory: 1126.0 → 1000.0 GJ/million_pkm.

## SAF blend

State ID: domestic_aviation__saf_blend. Classification: progression. Max share range: 2% (2025) to 60% (2050). Cost: AUD 0.08/million_pkm (2024 real). Energy trajectory: ATF+SAF blend 5%→100% SAF.

## Battery-electric short-haul

State ID: domestic_aviation__electric_short_haul. Classification: progression. Max share range: 0% (2025) to 20% (2050). Cost: AUD 0.12/million_pkm (2024 real). Energy trajectory: 360.0 → 280.0 GJ/million_pkm.

# 3. Key Assumptions

Demand anchor: 75,500 million_pkm (BITRE Aviation Statistical Report 2023-24)

Energy intensity: 85 PJ / 75,500 million_pkm = 1,126 GJ/million_pkm (AES 2023-24)

100% aviation turbine fuel in incumbent state

SAF biogenic CO2 excluded from scope 1 per NGA accounting

Electricity-related emissions excluded per A003 (scope 1 only)

GWP: IPCC AR5 GWP100

# 4. Known Caveats

National average masks route-length variation (short-haul vs long-haul domestic)

SAF supply chain for Australia is nascent; 100% SAF by 2050 is ambitious

Electric aviation limited to <500 km routes; max share 20% reflects route constraints

NGGI Cat 1A3a total 6.1 MtCO2e assumes all domestic fuel combustion in scope

# 5. Data Sources

S001 — AES 2023-24 Table F

S004 — National Inventory Report 2023

S012 — BITRE Aviation Statistical Report 2023-24

A002 — Cost definition: non-commodity conversion cost only

A003 — Scope 1 boundary: electricity-related emissions excluded

A022 — Feasibility bounds: indicative upper envelopes, not diffusion targets

A023 — Cost trajectories smoothed where evidence is sparse