# Domestic Shipping — Phase 1 Calibration Report

Family: domestic_shipping   |   Generated: 2026-05-02   |   Unit: million_tkm

# 1. Calibration Basis

Demand anchor. The 2025 baseline demand is anchored at 30,000 million_tkm.

Energy calibration. Total final energy at the 2025 anchor is 30.0 PJ (AES 2023-24). This gives an energy intensity of 1,000.0 GJ/million_tkm. Input fuels: marine_diesel_oil, heavy_fuel_oil.

Emissions calibration. Scope 1 CO2e at the anchor equals 2.28 MtCO2e (Domestic navigation). This uses NGA 2024 emission factors and IPCC AR5 GWP100. Electricity-related emissions are excluded (scope 2 boundary, per A003).

# 2. Technology States

## Conventional diesel

State ID: domestic_shipping__conventional_diesel. Classification: incumbent. Max share range: 100% (2025) to 65% (2050). Cost: AUD 2.5/million_tkm (2024 real). Energy trajectory: MDO 650 → 580 + HFO 350 → 310 GJ/million_tkm.

## Battery-electric vessel

State ID: domestic_shipping__battery_electric_vessel. Classification: progression. Max share range: 1% (2025) to 25% (2050). Cost: AUD 5.0/million_tkm (2024 real). Energy trajectory: 450 → 380 GJ electricity/million_tkm.

## Green ammonia vessel

State ID: domestic_shipping__green_ammonia_vessel. Classification: progression. Max share range: 0% (2025) to 20% (2050). Cost: AUD 8.0/million_tkm (2024 real). Energy trajectory: 1200 → 1000 GJ ammonia/million_tkm.

# 3. Key Assumptions

Demand anchor: 30,000 million_tkm = 30 billion tkm (BITRE coastal freight)

Energy intensity: 30 PJ / 30,000 million_tkm = 1,000 GJ/million_tkm

Fuel split: 65% MDO, 35% HFO (vessel type mix)

Green ammonia combustion CO2 = 0 (N2O slip excluded Phase 1)

Electric vessel scope 1 CO2 = 0; electricity scope 2 excluded per A003

GWP: IPCC AR5 GWP100

# 4. Known Caveats

Coastal navigation is heterogeneous; 65/35 MDO/HFO split is an approximation

N2O emissions from ammonia combustion excluded in Phase 1 (add in Phase 2)

Battery-electric range limits applicability to <100 km coastal routes

Green ammonia supply chain is nascent; max 20% by 2050 is exploratory

# 5. Data Sources

S001 — AES 2023-24 Table F (ANZSIC 4800)

S004 — National Inventory Report 2023

S013 — BITRE coastal freight statistics

A002 — Cost definition: non-commodity conversion cost only

A003 — Scope 1 boundary: electricity-related emissions excluded

A022 — Feasibility bounds: indicative upper envelopes, not diffusion targets

A023 — Cost trajectories smoothed where evidence is sparse