# International Aviation — Phase 1 Calibration Report

Family: international_aviation   |   Generated: 2026-05-02   |   Unit: million_pkm

# 1. Calibration Basis

Demand anchor. The 2025 baseline demand is anchored at 70,000 million_pkm.

Energy calibration. Total final energy at the 2025 anchor is 165.0 PJ (AES 2023-24). This gives an energy intensity of 2,357.0 GJ/million_pkm. Input fuels: aviation_turbine_fuel.

Emissions calibration. Scope 1 CO2e at the anchor equals 11.8 MtCO2e (International aviation bunkers). This uses NGA 2024 emission factors and IPCC AR5 GWP100. Electricity-related emissions are excluded (scope 2 boundary, per A003).

# 2. Technology States

## Conventional jet

State ID: international_aviation__conventional_jet. Classification: incumbent. Max share range: 100% (2025) to 70% (2050). Cost: AUD 0.04/million_pkm (2024 real). Energy trajectory: 2357.0 → 2100.0 GJ/million_pkm.

## SAF blend

State ID: international_aviation__saf_blend. Classification: progression. Max share range: 0% (2025) to 50% (2050). Cost: AUD 0.07/million_pkm (2024 real). Energy trajectory: ATF 100%→20% + SAF 0%→80%.

## Hydrogen aircraft

State ID: international_aviation__hydrogen_aircraft. Classification: progression. Max share range: 0% (2025) to 15% (2050). Cost: AUD 0.15/million_pkm (2024 real). Energy trajectory: 0 → 2200 → 2000 GJ H2/million_pkm (from 2035).

# 3. Key Assumptions

Demand anchor: 70,000 million_pkm (BITRE international air travel 2023-24)

Energy intensity: 165 PJ / 70,000 million_pkm = 2,357 GJ/million_pkm

International aviation bunkers (fuel uplifted in Australia)

SAF biogenic CO2 excluded from scope 1 per NGA

Hydrogen aircraft available from 2035 only

GWP: IPCC AR5 GWP100

# 4. Known Caveats

International bunker allocation has boundary uncertainty (vs ICAO fuel uplift data)

SAF penetration slower than domestic due to supply constraints for long-haul

H2 aircraft technology is highly uncertain; max 15% by 2050 is exploratory

Methane slip for LNG not modelled in Phase 1

# 5. Data Sources

S001 — AES 2023-24 international aviation bunkers

S004 — National Inventory Report 2023

S012 — BITRE international air travel 2023-24

A002 — Cost definition: non-commodity conversion cost only

A003 — Scope 1 boundary: electricity-related emissions excluded

A022 — Feasibility bounds: indicative upper envelopes, not diffusion targets

A023 — Cost trajectories smoothed where evidence is sparse