# International Shipping — Phase 1 Calibration Report

Family: international_shipping   |   Generated: 2026-05-02   |   Unit: million_tkm

# 1. Calibration Basis

Demand anchor. The 2025 baseline demand is anchored at 200,000 million_tkm.

Energy calibration. Total final energy at the 2025 anchor is 65.0 PJ (AES 2023-24). This gives an energy intensity of 325.0 GJ/million_tkm. Input fuels: heavy_fuel_oil, marine_diesel_oil.

Emissions calibration. Scope 1 CO2e at the anchor equals 5.0 MtCO2e (International maritime bunkers). This uses NGA 2024 emission factors and IPCC AR5 GWP100. Electricity-related emissions are excluded (scope 2 boundary, per A003).

# 2. Technology States

## Conventional HFO/MDO

State ID: international_shipping__conventional_hfo. Classification: incumbent. Max share range: 100% (2025) to 55% (2050). Cost: AUD 1.5/million_tkm (2024 real). Energy trajectory: HFO 195 → 165 + MDO 130 → 110 GJ/million_tkm.

## LNG transition

State ID: international_shipping__lng_transition. Classification: progression. Max share range: 1% (2025) to 30% (2050). Cost: AUD 1.8/million_tkm (2024 real). Energy trajectory: 350 → 290 GJ LNG/million_tkm.

## Zero-emission fuel (ammonia)

State ID: international_shipping__zero_emission_fuel. Classification: progression. Max share range: 0% (2025) to 25% (2050). Cost: AUD 4.0/million_tkm (2024 real). Energy trajectory: 0 → 400 → 320 GJ ammonia/million_tkm (from 2030).

# 3. Key Assumptions

Service volume 200,000 million_tkm is estimated (no direct Australian statistic)

Energy intensity: 65 PJ / 200,000 million_tkm = 325 GJ/million_tkm

Fuel split: 60% HFO, 40% MDO for international large vessels

Zero-emission fuel state available from 2030 only

Methane slip from LNG excluded in Phase 1

GWP: IPCC AR5 GWP100

# 4. Known Caveats

Service volume (200,000 million_tkm) is estimated; cross-check with IMO GHG data in Phase 2

LNG methane slip can increase net GHG relative to HFO; excluded Phase 1

N2O from ammonia combustion excluded Phase 1

International bunker data has flag/uplift-location boundary uncertainty

# 5. Data Sources

S001 — AES 2023-24 international maritime bunkers

S004 — National Inventory Report 2023

S013 — BITRE freight statistics

A002 — Cost definition: non-commodity conversion cost only

A003 — Scope 1 boundary: electricity-related emissions excluded

A022 — Feasibility bounds: indicative upper envelopes, not diffusion targets

A023 — Cost trajectories smoothed where evidence is sparse