# Rail Freight — Phase 1 Calibration Report

Family: rail_freight   |   Generated: 2026-05-02   |   Unit: billion_tkm

# 1. Calibration Basis

Demand anchor. The 2025 baseline demand is anchored at 700 billion_tkm.

Energy calibration. Total final energy at the 2025 anchor is 25.0 PJ (AES 2023-24). This gives an energy intensity of 35,715.0 GJ/billion_tkm. Input fuels: diesel, electricity.

Emissions calibration. Scope 1 CO2e at the anchor equals 1.62 MtCO2e (Rail freight scope 1 diesel). This uses NGA 2024 emission factors and IPCC AR5 GWP100. Electricity-related emissions are excluded (scope 2 boundary, per A003).

# 2. Technology States

## Diesel-electric locomotive

State ID: rail_freight__diesel_electric. Classification: incumbent. Max share range: 100% (2025) to 60% (2050). Cost: AUD 0.015/billion_tkm (2024 real). Energy trajectory: Diesel 33929 → 28500 + Elec 1786 → 1500 GJ/billion_tkm.

## Overhead electrification

State ID: rail_freight__overhead_electrification. Classification: progression. Max share range: 1% (2025) to 30% (2050). Cost: AUD 0.02/billion_tkm (2024 real). Energy trajectory: 25000 → 20000 GJ electricity/billion_tkm.

## Hydrogen locomotive

State ID: rail_freight__hydrogen_locomotive. Classification: progression. Max share range: 0% (2025) to 20% (2050). Cost: AUD 0.04/billion_tkm (2024 real). Energy trajectory: 0 → 40000 → 30000 GJ H2/billion_tkm (from 2030).

# 3. Key Assumptions

Demand anchor: 700 billion_tkm (BITRE freight linehaul; mainly iron ore and coal)

AES rail freight energy estimated at ~25 PJ (total rail ~33 PJ; freight share ~76%)

95% diesel, 5% electricity (electrified corridor fraction)

Scope 1 CO2e from diesel only; electricity scope 2 excluded per A003

Hydrogen locomotives available from 2030 only

GWP: IPCC AR5 GWP100

# 4. Known Caveats

AES does not separately report freight vs passenger rail; 25 PJ is an estimate

Heavy-haul iron ore (Pilbara private rail) dominates volumes but is separately operated

OHL electrification is very capital-intensive (AUD 1-3M/km); limited to high-density corridors

H2 locomotive technology is at demonstration scale; max 20% by 2050 is exploratory

# 5. Data Sources

S001 — AES 2023-24 estimated rail freight share

S004 — National Inventory Report 2023

S013 — BITRE freight linehaul statistics

A002 — Cost definition: non-commodity conversion cost only

A003 — Scope 1 boundary: electricity-related emissions excluded

A022 — Feasibility bounds: indicative upper envelopes, not diffusion targets

A023 — Cost trajectories smoothed where evidence is sparse