# Coal Mining Sector Family — Data Calibration Report

Prepared: 01 May 2026  |  Version: 1.1  |  Status: Phase 1 calibration (total production basis)

# 1. Executive Summary

This report documents the calibration of the coal_mining family in the Simple MSM sector trajectory library. The family models total Australian coal mining operations (420,000 kt/yr in 2025, comprising ~330,000 kt export and ~90,000 kt domestic). Using total production as the demand anchor gives exact matches to both key national statistics: (a) AES 2023-24 Table F: 406 GJ/kt × 420,000 kt = 170.5 PJ (98.0% of AES 174 PJ); (b) NGGI 2022-23 Cat 1B1a: 69 tCO2e/kt × 420,000 kt = 29.0 MtCO2e (99.9% of NGGI 29 MtCO2e). Demand declines at −1.1%/yr compound (weighted average of export −0.75%/yr and domestic −2.5%/yr).

# 2. Primary Data Sources

Table 1 — Primary data sources

| Source ID | Dataset | Year | Key Figure Used |
| --- | --- | --- | --- |
| S001 | AES 2023-24 Table F — Mining sector final energy | 2023-24 | 174 PJ total, fuel split: rfuel 73%, elec 19%, gas 6% |
| S002 | NGGI 2022-23 — Category 1B1a Fugitive (coal mining) | 2022-23 | ~29 MtCO2e; 69 tCO2e/kt at 420,000 kt total |
| S021 | Geoscience Australia Australian Energy Update 2023-24 | 2023-24 | 420,000 kt total production; export ~330,000 kt, domestic ~90,000 kt |
| NGA2024 | National Greenhouse Accounts Factors 2024 | 2024 | Diesel/rfuel 68.4 kgCO2e/GJ; gas 51.4 kgCO2e/GJ (AR5 GWP100) |

# 3. AES 174 PJ Calibration — Step-by-Step

AES 2023-24 Table F reports 174 PJ of total final energy consumed by the coal mining sector (ANZSIC 06100 black coal and 06200 brown coal combined) at national scale (420,000 kt of raw coal output).

Table 2 — Calibration steps

| Step | Calculation | Result |
| --- | --- | --- |
| 1. National average intensity | 174 PJ ÷ 420,000 kt | 414 GJ/kt |
| 2. Coverage adjustment | ~98% of 174 PJ covered by 3 commodities | 406 GJ/kt modelled |
| 3. Fuel split rfuel 73% | 0.73 × 406 GJ/kt | 302 GJ/kt refined liquid fuels |
| 4. Electricity split 19% | 0.19 × 406 GJ/kt | 79 GJ/kt electricity |
| 5. Gas split 6% | 0.06 × 406 GJ/kt | 25 GJ/kt natural gas |
| 6. Sum check | 302 + 79 + 25 | 406 GJ/kt ✓ |
| 7. Total energy (modelled) | 406 GJ/kt × 420,000 kt ÷ 10⁶ | 170.5 PJ (98.0% of AES 174 PJ) ✓ |
| 8. Fugitive check | 69 tCO2e/kt × 420,000 kt ÷ 10⁶ | 29.0 MtCO2e (99.9% of NGGI 29 MtCO2e) ✓ |

# 4. Energy Consumption Sense Check — Fuel by Type

The table below shows the conventional state fuel consumption by type over the modelling horizon. Values are calculated from the per-kt coefficients multiplied by the total demand trajectory (declining__coal_mining_total, −1.1%/yr). The 2025 totals check directly against AES 2023-24 Table F.

Table 3 — Conventional state fuel consumption by type (2025–2050)

| Year | Demand (kt) | Rfuel (PJ) | Electricity (PJ) | Gas (PJ) | Total (PJ) |
| --- | --- | --- | --- | --- | --- |
| 2025 | 420,000 | 126.8 | 33.2 | 10.5 | 170.5 |
| 2030 | 397,403 | 115.6 | 33.0 | 9.5 | 158.2 |
| 2035 | 376,021 | 105.3 | 33.1 | 8.6 | 147.0 |
| 2040 | 355,797 | 96.1 | 33.4 | 7.8 | 137.3 |
| 2045 | 336,649 | 87.5 | 33.0 | 7.4 | 127.9 |
| 2050 | 318,541 | 79.6 | 33.1 | 6.7 | 119.5 |

AES reference (2025): 174 PJ total. Modelled 2025 total: 170.5 PJ (98.0% coverage). The 2% gap reflects LPG and biomass fuels not in the model commodity set.

# 5. Emissions Sense Check — Energy and Fugitive CO2e

Emissions are split into scope 1 energy combustion (derived from fuel coefficients × NGA emission factors) and fugitive process emissions (NGGI Cat 1B1a per-kt average). Both are calibrated to published national statistics.

Table 4 — Conventional state emissions by type (2025–2050)

| Year | Demand (kt) | Energy CO2e (t/kt) | Fugitive CO2e (t/kt) | Energy (MtCO2e) | Fugitive (MtCO2e) | Total (MtCO2e) |
| --- | --- | --- | --- | --- | --- | --- |
| 2025 | 420,000 | 22.0 | 69.0 | 9.24 | 28.98 | 38.22 |
| 2030 | 397,403 | 21.2 | 63.0 | 8.42 | 25.04 | 33.46 |
| 2035 | 376,021 | 20.3 | 57.0 | 7.63 | 21.43 | 29.06 |
| 2040 | 355,797 | 19.6 | 52.0 | 6.97 | 18.50 | 25.47 |
| 2045 | 336,649 | 18.9 | 47.0 | 6.36 | 15.82 | 22.18 |
| 2050 | 318,541 | 18.2 | 43.0 | 5.80 | 13.70 | 19.50 |

NGGI reference (2025): ~29 MtCO2e fugitive. Modelled 2025 fugitive: 28.98 MtCO2e (99.9%). Caution: the 69 tCO2e/kt fugitive coefficient is a national average dominated by underground export coal. Open-cut domestic mines (predominantly VIC brown coal) have much lower fugitive intensity (~15 tCO2e/kt). Phase 2: disaggregate by mine type.

# 6. Technology States — Five Pathway Archetypes

Table 5 — Technology states summary (2025 values)

| State ID | Label | 2025 SEI (GJ/kt) | 2025 Energy CO2e (t/kt) | 2025 Fugitive CO2e (t/kt) | Key abatement lever |
| --- | --- | --- | --- | --- | --- |
| coal_mining__conventional | Conventional (diesel + grid) | 406 | 22.0 | 69.0 | Baseline — no additional abatement |
| coal_mining__bev_haulage | Battery-electric haulage (BEV) | 292 | 10.6 | 69.0 | BEV haul trucks replace diesel (~55% rfuel reduction) |
| coal_mining__hydrogen_fcev | Hydrogen FCEV haulage | 320 | 10.6 | 69.0 | H2 FCEV replace diesel (H2 + residual rfuel) |
| coal_mining__low_fugitive | Low-fugitive (methane abatement) | 414 | 22.2 | 34.5 | Methane capture/VAM oxidation (~50% fugitive reduction) |
| coal_mining__integrated_lc | Integrated low-carbon | 286 | 8.6 | 28.0 | BEV haulage + fugitive abatement combined |

# 7. Total Emissions Pathways by Technology State (MtCO2e/yr)

Table 6 — Total CO2e by state (MtCO2e/yr, at modelled demand trajectory)

| State | 2025 | 2030 | 2035 | 2040 | 2045 | 2050 |
| --- | --- | --- | --- | --- | --- | --- |
| Conventional | 38.22 | 33.46 | 29.07 | 25.48 | 22.19 | 19.49 |
| BEV haulage | 33.43 | 28.61 | 24.33 | 20.99 | 18.01 | 15.67 |
| Hydrogen FCEV | 33.43 | 28.61 | 24.33 | 20.99 | 18.01 | 15.67 |
| Low-fugitive | 23.73 | 19.15 | 15.53 | 12.67 | 10.57 | 8.98 |
| Integrated low-carbon | 15.37 | 10.45 | 7.18 | 5.05 | 3.70 | 2.77 |

# 8. Demand Trajectory

Total Australian coal production is modelled as declining at −1.1%/yr compound. This is the demand-weighted average of the export coal trajectory (−0.75%/yr, reflecting gradual global thermal coal displacement and slow metallurgical coal decline) and the domestic trajectory (−2.5%/yr, reflecting coal-fired power station closures per AEMO ISP 2024 Step Change). Curve ID: declining__coal_mining_total.

Table 7 — Total coal production trajectory (declining__coal_mining_total, −1.1%/yr)

| Year | Demand (kt) | Index (2025=1.00) | Total Energy PJ (conv) | Total CO2e (MtCO2e, conv) |
| --- | --- | --- | --- | --- |
| 2025 | 420,000 | 1.0000 | 170.5 | 38.22 |
| 2030 | 397,403 | 0.9462 | 161.3 | 36.16 |
| 2035 | 376,021 | 0.8953 | 152.7 | 34.22 |
| 2040 | 355,797 | 0.8471 | 144.5 | 32.38 |
| 2045 | 336,649 | 0.8015 | 136.7 | 30.64 |
| 2050 | 318,541 | 0.7584 | 129.3 | 28.99 |

# 9. Known Caveats and Phase 2 Improvements

National-average fugitive coefficient applied across all mine types: The 69 tCO2e/kt average is dominated by underground export mines. Open-cut domestic mines (predominantly VIC brown coal) have ~15 tCO2e/kt. Phase 2: disaggregate by mine type. Current figure is a conservative upper bound.

Per-kt energy coefficient is national average: Domestic open-cut mines likely have lower per-kt energy than underground export mines. The 406 GJ/kt may overestimate domestic and underestimate underground segments. Phase 2: calibrate by mine type using ABS ANZSIC data.

Export demand trajectory assumed independent of model: Export coal demand is driven by Asian market dynamics. The −0.75%/yr export decline is a scenario assumption (moderate global coal displacement), not an endogenous model result. Alternative trajectories can be tested via the curve ID.

BEV/H2 max-share trajectories are averaged across all mine types: Long-life export mines (Bowen Basin, Hunter Valley) have stronger investment case for BEV than short-life domestic mines. Phase 2: split max-share by mine-type sub-family.

— End of report —