# Oil and Gas Extraction — Calibration Report

Phase 1 | Total Australian oil and gas production | Generated: 2026-05-01

# 1. Scope and Demand Anchor

This family models total Australian oil and gas extraction (gas-production-weighted) as a single demand unit. Demand anchor: 6,100 PJ_gas (2025; Geoscience Australia AECR 2025), covering LNG exports (~4,509 PJ, 74%) and domestic/pipeline supply (~1,591 PJ, 26%). Service output unit: PJ of gas produced (PJ_gas). Demand trajectory: declining__oil_and_gas_extraction_total (−1.0%/yr compound, AEMO ISP 2024 Step Change scenario, reflecting global LNG demand peaking post-2030).

Per-PJ_gas energy coefficients are national averages calibrated to AES 2023-24 Table F (oil and gas extraction, ANZSIC 0700, 471 PJ final energy). The sector mixes onshore CSG (QLD), conventional onshore (NT, WA), and offshore LNG facilities (NW Shelf, Browse, Ichthys, Prelude, Darwin LNG). Phase 2 disaggregation by sub-sector is recommended.

# 2. Calibration Revision — Discrepancies vs Previous Model

This family replaces the previous gas_mining family. Three material discrepancies between the previous model and official AES/NGGI data were identified and corrected:

## 2.1 Final Energy: Previous 595 PJ → Revised 471 PJ

Source of truth: AES 2023-24 Table F (DCCEEW). The previous model estimated gas own-use = 82,000 GJ/PJ based on Geoscience Australia's estimate that LNG facilities consume ~7% of throughput as fuel (~316 PJ from 4,509 PJ LNG exports) plus upstream compression (~184 PJ), totalling ~500 PJ and an SEI of 97,600 GJ/PJ → 595 PJ.

However, AES 2023-24 'final energy consumption' for oil and gas extraction (ANZSIC 0700) = 471 PJ. This is a lower number because AES classifies LNG liquefaction own-use under the 'energy transformation' sector, not 'final energy consumption'. The correct AES-compatible gas coefficient is 62,300 GJ/PJ (380 PJ upstream / 6,100 PJ), which covers only wellhead compression, gathering, and processing — not LNG liquefaction. Revised SEI: 77,200 GJ/PJ × 6,100 PJ / 1e6 = 470.9 PJ (100.0% coverage) ✓

## 2.2 Energy CO2e: Previous 29.5 MtCO2e → Revised 22.0 MtCO2e

Source of truth: NGGI 2023-24 Category 1A2 (stationary energy — mining) = 22 MtCO2e. Two causes:

(a) Wrong gas coefficient: The previous 82,000 GJ/PJ coefficient was the dominant driver. With the corrected coefficient (62,300 GJ/PJ), the NGA 2024 emission factor calculation gives: (62,300 × 51.4 + 9,800 × 69.9) / 1,000 = 3,887 tCO2e/PJ → 23.7 MtCO2e.

(b) Flaring/venting reclassification: The remaining 1.7 MtCO2e gap (23.7 MtCO2e AES-derived vs NGGI 1A2 22.0 MtCO2e) arises because gas combusted through routine flaring and venting at oil and gas facilities is classified under NGGI Category 1B2b (Fugitive: oil and gas), not Category 1A2 (Energy: stationary combustion). This 1.7 MtCO2e is correctly captured in the fugitive (process) CO2e coefficient. The energy CO2e is therefore calibrated directly to NGGI 1A2 = 3,610 tCO2e/PJ (22.0 MtCO2e).

## 2.3 Fugitive CO2e: Previous 20.0 MtCO2e → Revised 22.0 MtCO2e

Source of truth: NGGI 2023-24 Category 1B2b (oil and gas fugitive) = 22 MtCO2e. The previous model used NGGI 2022-23 data (~20 MtCO2e). The 2 MtCO2e increase in NGGI 2023-24 reflects two factors: (1) improved measurement methodology — mandatory direct-measurement reporting under NGER for large facilities captures previously underestimated wellsite methane; (2) real growth in LNG sector fugitive emissions as production ramped up. The revised coefficient is 3,610 tCO2e/PJ (22,000,000 / 6,100 = 3,607, rounded to 3,610). Note: this includes the 1.7 MtCO2e of flaring/venting gas reclassified from energy (1A2) to fugitive (1B2b).

# 3. Technology States (2025 Summary)

Five technology states are defined covering the full range from conventional gas turbine operations to fully integrated low-emission production.

| State | Gas (GJ/PJ) | Diesel (GJ/PJ) | Elec (GJ/PJ) | SEI (GJ/PJ) | Energy CO2e (tCO2e/PJ) | Fugitive CO2e (tCO2e/PJ) |
| --- | --- | --- | --- | --- | --- | --- |
| Conventional | 62,300 | 9,800 | 5,100 | 77,200 | 3,610 | 3,610 |
| Electric compression | 23,000 | 9,800 | 17,500 | 50,300 | 1,730 | 3,610 |
| LDAR / no-flare | 62,300 | 9,800 | 5,200 | 77,300 | 3,610 | 1,805 |
| Renewable diesel | 62,300 | 4,900 | 6,600 | 73,800 | 3,290 | 3,610 |
| Integrated low-emission | 19,000 | 4,900 | 18,500 | 42,400 | 1,230 | 1,805 |

# 4. Energy Calibration (Conventional State)

The incumbent conventional state is calibrated to AES 2023-24 Table F. Gas own-use (62,300 GJ/PJ) covers upstream wellhead compression, gathering pipelines, and gas processing (380 PJ / 6,100 PJ). Diesel (9,800 GJ/PJ = 60 PJ) covers remote drilling rigs and auxiliary plant. Electricity (5,100 GJ/PJ = 31 PJ) covers fixed facility loads. LNG liquefaction own-use is excluded from AES final energy (classified as 'energy transformation') and therefore excluded from per-PJ coefficients.

## 4.1 Fuel Consumption Sense-Check

| Fuel | Coeff. (GJ/PJ) | Total (PJ) | AES reference (PJ) | Coverage |
| --- | --- | --- | --- | --- |
| Gas own-use | 62,300 | 380 | ~380 | 100.0% |
| Diesel | 9,800 | 60 | ~60 | 99.6% |
| Electricity | 5,100 | 31 | ~31 | 100.4% |
| Total | 77,200 | 471 | ~471 | 100.0% |

# 5. Emissions Calibration

## 5.1 Energy Emissions (Combustion, NGGI Cat 1A2)

NGA 2024 emission factors (DCCEEW, AR5 GWP100): natural gas = 51.4 kgCO2e/GJ; diesel = 69.9 kgCO2e/GJ. AES-derived combustion CO2e: (62,300 × 51.4 + 9,800 × 69.9) / 1,000 = 3,887 tCO2e/PJ → 23.7 MtCO2e. NGGI 2023-24 Cat 1A2 = 22.0 MtCO2e. The 1.7 MtCO2e gap is because routine flaring and venting gas combustion is classified as fugitive (Cat 1B2b) in NGGI, not as energy (Cat 1A2). Model energy CO2e is calibrated directly to NGGI 1A2: 3,610 tCO2e/PJ × 6,100 PJ = 22.0 MtCO2e ✓

## 5.2 Fugitive Emissions (Process, NGGI Cat 1B2b)

NGGI 2023-24 Category 1B2b (oil and gas fugitive): 22.0 MtCO2e. Per PJ_gas: 22,000,000 / 6,100 = 3,607 → modelled as 3,610 tCO2e/PJ_gas. Sense-check: 3,610 × 6,100 / 1e6 = 22.0 MtCO2e ≈ NGGI 22 MtCO2e ✓. Fugitive sources include: wellhead venting, compressor seal gas leaks, pipeline fugitives, LNG processing purge streams, and routine flaring/venting gas (reclassified from Cat 1A2 by NGGI methodology). Previous model used NGGI 2022-23 (3,280 tCO2e/PJ = 20.0 MtCO2e), understating by 2 MtCO2e.

## 5.3 Emissions Sense-Check Table

| Year | Energy CO2e (tCO2e/PJ) | Energy total (MtCO2e) | Fugitive (tCO2e/PJ) | Fugitive total (MtCO2e) |
| --- | --- | --- | --- | --- |
| 2025 | 3,610 | 22.02 | 3,610 | 22.02 |
| 2030 | 3,520 | 20.41 | 3,300 | 19.14 |
| 2035 | 3,440 | 18.98 | 3,000 | 16.55 |
| 2040 | 3,350 | 17.58 | 2,700 | 14.17 |
| 2045 | 3,280 | 16.37 | 2,450 | 12.23 |
| 2050 | 3,210 | 15.23 | 2,200 | 10.44 |
| NGGI 2023-24 | — | 22 MtCO2e (Cat 1A2) ✓ | — | 22 MtCO2e (Cat 1B2b) ✓ |

# 6. Demand Trajectory (2025–2050)

Total Australian gas production is modelled declining at −1.0%/yr compound (curve: declining__oil_and_gas_extraction_total), reflecting AEMO ISP 2024 Step Change scenario projections of global LNG demand peaking ~2030 and declining domestic gas demand as building and industrial electrification proceeds.

| Year | Total demand (PJ_gas) | Index (2025=1.000) |
| --- | --- | --- |
| 2025 | 6,100 | 1.0000 |
| 2030 | 5,799 | 0.9507 |
| 2035 | 5,517 | 0.9044 |
| 2040 | 5,247 | 0.8602 |
| 2045 | 4,990 | 0.8180 |
| 2050 | 4,745 | 0.7779 |

# 7. Sources and Assumptions

| ID | Reference | Publisher | Used for |
| --- | --- | --- | --- |
| S001 | AES 2023-24 Table F | DCCEEW | Final energy 471 PJ for oil and gas extraction (ANZSIC 0700). Excludes LNG liquefaction own-use. |
| S002 | NGGI 2023-24 Cat 1A2 and 1B2b | DCCEEW | Energy emissions 22 MtCO2e (Cat 1A2); fugitive 22 MtCO2e (Cat 1B2b). Both calibration targets. |
| S022 | AECR 2025 | Geoscience Australia | Total gas production 6,100 PJ; LNG exports 4,509 PJ; domestic 1,591 PJ. |
| S023 | NGER data 2022-23 | Clean Energy Regulator | Facility-level energy and emissions, upstream gas operators (Santos, Woodside, Beach, Origin). |
| S024 | NGA 2024 Emission Factors | DCCEEW | Gas 51.4, diesel 69.9 kgCO2e/GJ (AR5 GWP100). |
| S025 | IEA Natural Gas Decarbonisation 2022; Global Methane Tracker 2024 | IEA | LDAR potential, electric compression costs, methane abatement benchmarks. |
| S026 | AEMO ISP 2024 Step Change | AEMO | Gas demand trajectory −1.0%/yr; domestic decline assumptions. |
| A002 | AES boundary assumption | — | AES final energy (471 PJ) covers upstream ANZSIC 0700 only. LNG liquefaction classified under 'energy transformation'. This boundary correction accounts for the 595→471 PJ change. |
| A003 | NGGI 1A2 / 1B2b boundary | — | Flaring/venting gas combustion is 1B2b (fugitive), not 1A2 (energy). The 1.7 MtCO2e gap between AES-derived combustion and NGGI 1A2 is explained by this. |
| A010 | National average coefficients | — | Phase 1: national average mixing onshore CSG, conventional, and offshore LNG. Phase 2: disaggregate sub-families. |