# Non-Road Transport Sector — Recalibration Report

AES 2025 Table F1 (2023-24 column) recalibration  |  2026-05-05

# 1. Overview

This report documents the recalibration of the six non-road transport families (domestic and international aviation, domestic and international shipping, and rail passenger and rail freight) against the Australian Energy Statistics 2025 (AES 2025) Table F1, using the 2023-24 column. The recalibration replaces the prior AES 2023-24 calibration that had been applied previously. Service-demand anchors (passenger-km, tonne-km) are unchanged; only the per-unit energy intensity coefficients in each family's incumbent state are updated. Transition-state intensities (SAF blends, fuel-cell aircraft, battery-electric vessels, electrified rail) remain at their technology-specific values because they reflect engineering characteristics rather than the AES baseline.

# 2. Calibration summary (AES 2025 Table F1, 2023-24)

| Family | Anchor | Total energy | Total scope 1 CO2e |
| --- | --- | --- | --- |
| Domestic aviation | 75,500 million pkm | 135.3 PJ | 9.67 MtCO2e (scope 1) |
| International aviation | 70,000 million pkm | 199.0 PJ | 14.2 MtCO2e (scope 1, bunkers) |
| Domestic shipping (coastal) | 30,000 million tkm | 44.5 PJ | 3.38 MtCO2e (scope 1) |
| International shipping | 200,000 million tkm (estimated) | 29.5 PJ | 2.3 MtCO2e (scope 1, bunkers) |
| Rail passenger | 22,000 million pkm | 16.0 PJ | 0.22 MtCO2e (scope 1 diesel only) |
| Rail freight | 700 billion tkm | 49.0 PJ | 3.43 MtCO2e (scope 1) |

# 3. Calibration changes vs prior AES 2023-24 calibration

| Family | Old AES PJ | New AES PJ | Source |
| --- | --- | --- | --- |
| Domestic aviation | 85.0 (AES 2023-24) | 135.3 (AES 2025 F1) | Revision: domestic air transport now reported separately at 135.3 PJ |
| International aviation | 165.0 (AES 2023-24) | 199.0 (AES 2025 F1) | Increase reflects post-COVID international travel rebound to 2023-24 |
| Domestic shipping | 30.0 (AES 2023-24) | 44.5 (AES 2025 F1) | Coastal bunkers re-stated higher in AES 2025 |
| International shipping | 65.0 (AES 2023-24) | 29.5 (AES 2025 F1) | International bunkers re-stated lower (boundary correction) |
| Rail passenger | 8.0 (AES 2023-24) | 16.0 (AES 2025 F1) | Reflects 12.9 PJ rail electricity + 3.1 PJ regional diesel |
| Rail freight | 25.0 (AES 2023-24) | 49.0 (AES 2025 F1) | Reflects 49 PJ rail diesel allocated to freight |

## 4. Key takeaways

Domestic aviation almost doubled (85 → 135.3 PJ); reflects the full reporting of domestic air transport in AES 2025 Table F1.

International aviation bunkers rose 165 → 199 PJ; consistent with post-COVID long-haul service recovery to/from Australia.

Coastal shipping rose 30 → 44.5 PJ; international bunkers fell 65 → 29.5 PJ. The international value drop is a boundary re-statement rather than a real decline.

Rail total (sector 47) is now 64.9 PJ (52 PJ diesel + 12.9 PJ electricity), almost double the prior 33 PJ aggregate. The split applied here allocates all rail electricity (12.9 PJ) plus 3.1 PJ regional diesel to passenger services (16 PJ), and the remaining 49 PJ diesel to freight.

# 5. Per-family calibration detail

## Domestic aviation

| Family ID | domestic_aviation |
| --- | --- |
| Service demand anchor (2025) | 75,500 million pkm |
| Demand growth curve | growing_then_flat__domestic_aviation |
| Energy intensity (incumbent) | 1,792 GJ/million_pkm (2025) → 1,592 (2050) |
| Fuel mix at 2025 | Aviation turbine fuel 100% |
| Total energy at anchor (AES 2025) | 135.3 PJ |
| AES source | AES 2025 Table F1, sector 49 'Of which domestic air transport' (2023-24) |
| Total scope 1 CO2e at anchor | 9.67 MtCO2e (scope 1) |
| NGGI cross-check | NGGI 2025 Cat 1A3a domestic aviation |

## International aviation

| Family ID | international_aviation |
| --- | --- |
| Service demand anchor (2025) | 70,000 million pkm |
| Demand growth curve | growing__international_aviation |
| Energy intensity (incumbent) | 2,843 GJ/million_pkm (2025) → 2,533 (2050) |
| Fuel mix at 2025 | Aviation turbine fuel 100% (bunker) |
| Total energy at anchor (AES 2025) | 199.0 PJ |
| AES source | AES 2025 Table F1, sector 49 'Of which international air transport' (2023-24) |
| Total scope 1 CO2e at anchor | 14.2 MtCO2e (scope 1, bunkers) |
| NGGI cross-check | NGGI 2025 international aviation bunkers |

## Domestic shipping (coastal)

| Family ID | domestic_shipping |
| --- | --- |
| Service demand anchor (2025) | 30,000 million tkm |
| Demand growth curve | stable__domestic_shipping |
| Energy intensity (incumbent) | 1,483 GJ/million_tkm (2025) → 1,320 (2050) |
| Fuel mix at 2025 | Marine diesel oil 65% / Heavy fuel oil 35% |
| Total energy at anchor (AES 2025) | 44.5 PJ |
| AES source | AES 2025 Table F1, sector 48 'Of which coastal bunkers' (2023-24) |
| Total scope 1 CO2e at anchor | 3.38 MtCO2e (scope 1) |
| NGGI cross-check | NGGI 2025 domestic navigation |

## International shipping

| Family ID | international_shipping |
| --- | --- |
| Service demand anchor (2025) | 200,000 million tkm (estimated) |
| Demand growth curve | growing__international_shipping |
| Energy intensity (incumbent) | 147.5 GJ/million_tkm (2025) → 125 (2050) |
| Fuel mix at 2025 | Heavy fuel oil 60% / Marine diesel oil 40% (bunker) |
| Total energy at anchor (AES 2025) | 29.5 PJ |
| AES source | AES 2025 Table F1, sector 48 'Of which international bunkers' (2023-24) |
| Total scope 1 CO2e at anchor | 2.3 MtCO2e (scope 1, bunkers) |
| NGGI cross-check | NGGI 2025 international maritime bunkers |

## Rail passenger

| Family ID | rail_passenger |
| --- | --- |
| Service demand anchor (2025) | 22,000 million pkm |
| Demand growth curve | growing__rail_passenger |
| Energy intensity (incumbent) | 727 GJ/million_pkm (2025) → 722 (2050) |
| Fuel mix at 2025 | Electricity 586 GJ (urban metro) / Diesel 141 GJ (regional) |
| Total energy at anchor (AES 2025) | 16.0 PJ |
| AES source | AES 2025 Table F1, sector 47 Rail electricity 12.9 PJ + regional diesel 3.1 PJ allocated to passenger (2023-24) |
| Total scope 1 CO2e at anchor | 0.22 MtCO2e (scope 1 diesel only) |
| NGGI cross-check | NGGI 2025 rail (passenger share) |

## Rail freight

| Family ID | rail_freight |
| --- | --- |
| Service demand anchor (2025) | 700 billion tkm |
| Demand growth curve | stable_growing__rail_freight |
| Energy intensity (incumbent) | 70,000 GJ/billion_tkm (2025) → 58,800 (2050) |
| Fuel mix at 2025 | Diesel 100% (heavy haul + interstate intermodal) |
| Total energy at anchor (AES 2025) | 49.0 PJ |
| AES source | AES 2025 Table F1, sector 47 Rail diesel 52 PJ minus 3.1 PJ regional passenger (2023-24) |
| Total scope 1 CO2e at anchor | 3.43 MtCO2e (scope 1) |
| NGGI cross-check | NGGI 2025 rail (freight share) |

# 6. Method notes

Demand anchors are unchanged because BITRE service volume statistics for passenger-km and tonne-km were not affected by the AES revision. Only per-unit energy intensity coefficients on the incumbent (conventional) state in each family were rescaled. Transition-state coefficients (SAF blends, hydrogen aircraft, battery vessels, electrified rail) preserve their technology-specific values because those reflect engineering characteristics rather than the AES baseline.

The rail passenger / freight split is not directly published in AES. The allocation applied here assigns the full 12.9 PJ rail electricity to passenger services (urban metro is electric-dominant) plus 3.1 PJ of regional diesel passenger services, totalling 16 PJ rail passenger. The remainder (49 PJ diesel) is allocated to rail freight. This carries calibration uncertainty until a Phase 2 disaggregation by service class.

# 7. Sources

S001 — Australian Energy Statistics 2025 (Table F1, 2023-24 column).

S004 — National Greenhouse Gas Inventory (NGGI) 2025.

S012 — BITRE Aviation Statistical Report 2023-24; Rail Summary Data 2023-24.

S013 — BITRE coastal freight statistics; freight linehaul.

S032 — NGA Factors 2025 (NGA 2024) — emission factors for diesel, jet fuel, MDO, HFO.

# 8. Files affected

sector_trajectory_library/families/domestic_aviation/generate_domestic_aviation_data.py

sector_trajectory_library/families/domestic_aviation/family_states.csv (regenerated)

sector_trajectory_library/families/domestic_aviation/demand.csv (regenerated)

sector_trajectory_library/families/domestic_aviation/README.md

sector_trajectory_library/families/international_aviation/generate_international_aviation_data.py

sector_trajectory_library/families/international_aviation/family_states.csv (regenerated)

sector_trajectory_library/families/international_aviation/demand.csv (regenerated)

sector_trajectory_library/families/international_aviation/README.md

sector_trajectory_library/families/domestic_shipping/generate_domestic_shipping_data.py

sector_trajectory_library/families/domestic_shipping/family_states.csv (regenerated)

sector_trajectory_library/families/domestic_shipping/demand.csv (regenerated)

sector_trajectory_library/families/domestic_shipping/README.md

sector_trajectory_library/families/international_shipping/generate_international_shipping_data.py

sector_trajectory_library/families/international_shipping/family_states.csv (regenerated)

sector_trajectory_library/families/international_shipping/demand.csv (regenerated)

sector_trajectory_library/families/international_shipping/README.md

sector_trajectory_library/families/rail_passenger/generate_rail_passenger_data.py

sector_trajectory_library/families/rail_passenger/family_states.csv (regenerated)

sector_trajectory_library/families/rail_passenger/demand.csv (regenerated)

sector_trajectory_library/families/rail_passenger/README.md

sector_trajectory_library/families/rail_freight/generate_rail_freight_data.py

sector_trajectory_library/families/rail_freight/family_states.csv (regenerated)

sector_trajectory_library/families/rail_freight/demand.csv (regenerated)

sector_trajectory_library/families/rail_freight/README.md

web/test/firstClassResiduals.test.mjs (electricity balance assertion updated)